CRH7300_TH01294 Buyoff List

This document regulates the verification items during examination and acceptance procedure. All requirements must be listed on this list. Requirements or features not included in this document should not be used as criteria for acceptance procedure.

Package & Test Scenario

The verification should be based on the following package and test cases.

| Item | Definition |
| --- | --- |
| Package | SSO6, Kelvin Contact |
| Carriage | Tube |

Basic Specification & Configuration

| Item |  | Desc. | Pass/Fail | Remark |
| --- | --- | --- | --- | --- |
|  | Machine Power & Electrical plug type | Verification procedures: Measure the facility voltage by multi-meter Electrical power: AC 220V/30A, 1Φ, 60Hz |  |  |
|  | IPCs Auto On | Verification procedures: After the handler IPC shuts down, the IPCs can be powered on automatically when handler is turned on. |  |  |
|  | OS | Specification: Windows 10 |  |  |
|  | Language, Region and Time Zone | Verification procedures: Open the system setting page and check whether it’s set accordingly. |  |  |
|  | Air Gauge Check | Verification procedures: Check the gauges on machine after the air connects to facility. The air pressure should be 5 kgf/cm2. |  |  |
|  | Vacuum Solenoid Valve Check | Verification procedures: - Connect the air source to the machine. - Check if the valve is working. |  |  |
|  | Mouse + Keyboard Function Check | Verification procedures: - Proceed machine initialization first. - Press Mouse/Keyboard and check if the device is working properly. |  |  |

Safety & Supporting

| Item |  | Desc. | Pass/Fail | Remark |
| --- | --- | --- | --- | --- |
|  | Safety cover Check | Verification procedure: - Check if the cover properly covers the sorting motor. |  |  |
|  | Emergency switch Function | Verification procedure: - Make sure the handler runs in normal mode. - Press the Emergency Stop button. - Check if all motors are stopped immediately. - Manually move the pickup head and check if it is locked. |  |  |
|  | Stack Light | Verification procedure: Check the light under the following circumstances. - Red Light: Severe Malfunction - Yellow Light: Alert - Green Light: Running |  |  |
|  | Manual | Verification procedure: Check if the following parts are included in the user manual. - User Manual. - Troubleshooting |  |  |
|  | Warranty | Specification: 1-year warranty Verification procedure: Check if the warranty certificate is provided. |  |  |
|  | Training | Requirements: - Initial setup at testing factory. - 8hrs operation training in factory. - 8hrs maintenance Training in factory. |  |  |

Machine Specification

| Item |  | Desc. | Pass/Fail | Remark |
| --- | --- | --- | --- | --- |
| 2 | Dimension | Specification: 700 (W) x 1220 (L) x 2100 (H) (Estimated) |  |  |
|  | Weight | Specification: 370 kg (Estimated) |  |  |
|  | Input Tube Loader | Specification: Auto loader x 1 Stack Capacity: 30 Tube |  |  |
|  | Output Tube Unloader | Specification: Auto Unloader x 1 Manual Unloader x 6 Stack Capacity: 30 Tube |  |  |
|  | UPH | Specification: Max. 6000; Dual Sites; Package: SSO6 Test time: 0 sec, Ambient. |  |  |
|  | Jam Rate | Specification: 1/3000 |  |  |
|  | Temperature | Specification: 55°C to 100°C (±2°C) 100°C to 125°C (±3°C) |  |  |
|  | Interface | Specification: GPIB&KVD |  |  |

Software Features

| Item |  | Desc. | Pass/Fail | Remark |
| --- | --- | --- | --- | --- |
|  | Security Management | Specification: - Account base management. - The password should be changeable. |  |  |
|  | Recipe (Job File) | Specification: Support at least 10 recipes. |  |  |
|  | Alert Message | Specification: The “alert prompt” should include guidance to resolve issues. |  |  |
|  | Error Logging & Profiling | Specification: - The handler should keep logs for at least 3.5 days. (Ex. Friday pm to Monday am) - The log needs to be able to be exported as Excel file. |  |  |
|  | End Lot | Specification: Create a lot report when lot is finished. |  |  |

Signature

| Customer | Buyoff | Installation |
| --- | --- | --- |

Date ____________